3GPP TSG SA WG3 LI Meeting  #11	S3LI20_111

Budapest, Hungary, 4 – 6 May 2002

Source:	Nokia

Title:	Implications of the IPv6 autoconfiguration on LI

Document for:	Discussion

References:	AP02-17; CR 286r4 against 23.060v5.0.0 (enclosed); CR 044r2 against 29.061v5.1.0 (enclosed).

Discussion:

SA plenary #15 has approved CR 286r4 to 23.060v5.0.0 in March 2002. CN3 has drafted a follow up CR 044r2 against 29.061v5.1.0 in April 2002. The latter CR goes to the next SA plenary in June 2002 for approval.

Those CRs introduce means to support IPv6 autoconfiguration feature in UMTS. This brings UMTS closer to the public Internet. Our task was to study possible implications of the new feature on the existing LI solutions.

Two types of address autoconfiguration were introduced: stateful and stateless.

First, we need to consider the following feature of the IPv6 protocol stack. Two parts make up a unicast IPv6 address: a Prefix, and an Interface ID.

In the stateful address autoconfiguration the following simplified sequence of events take place:

- APN is configured to perform stateful address autoconfiguration.

- A PDP context is activated. Naturally, this is reported to LEMF.

- UE and ISP/intranet negotiate new IPv6 address. Here, GGSN acts as a DHCP relay agent.

- Once new IPv6 address has been assigned to UE, GGSN initiates the PDP context modification procedure. Naturally, this is reported to LEMF.

Hence, stateful address autoconfiguration does not require special care to meet the LI requirements.

In the stateless address autoconfiguration the following simplified sequence of events take place:

- APN is configured to perform stateless address autoconfiguration.

- A PDP context is activated. Naturally, this is reported to LEMF.

- GGSN assigns an IPv6 address to UE. This address must have a globally unique prefix. Therefore, GGSN routs incoming packet to UE after looking only into the prefix, because of the following reason.

- UE is entitled to select arbitrary value for Interface ID. A possible intruder at the public Internet part of the path cannot know whether stateless autoconfiguration has been performed or not. Moreover, the attacker cannot know the length of a prefix in the given operator’s network. Therefore, the attacker cannot correlate IP packets even if they have same prefixes.

- UE never communicates new Interface ID to any of GSNs involved. Apparently the new Interface ID cannot be communicated further to LEMF. However, this is not needed either, because LEMF could identify the UE by the prefix.

Hence, in order to meet the LI requirements once stateful address autoconfiguration is performed, GGSN should report a new event. GGSN simply should send with an IRI the globally unique prefix of the UE’s IPv6 address to the LEMF via DF2.

Proposal:

It is proposed to discuss the matter and draft two CRs against 33.107 and 33.108.